# Lighting Design Basics

Based on the Book by Mark

Karlen and James Benya,

Wiley and Sons, 2004

Presented by

NKBA and

Benya Lighting

James Robert Benya, PE, FIES, IALD, LC

BENYA LIGHTING DESIGN

Portland, OR

No Handouts?!?!?

•Get the book

– www.wiley.com

•Download this presentation at www.benyalighting.com

•Questions?  Send a message to jbenya@benyalighting.com

What are lighting design basics?

- Introduction

- Light Sources

- Luminaires

- Switching and Dimming

- Daylighting

- Lighting Calculations

- Documenting Lighting Design

- The Layers Approach

- A Basic Approach

What are lighting design basics?

- Residential Lighting Design

- Office and Corporate Lighting Design

- Hospitality Lighting Design

- Health Care/Institutional Lighting Design

- Lighting for Stores

- Lighting Common Spaces

What are lighting design basics?

- The Professional Process of Lighting Design

- Collaborating with Lighting Designers

- Computers and Lighting Design

- Developing Skills Beyond the Basics

# 1  Introduction

A GOOD LIGHTING DESIGN SHOULD • Look good!

- Provide the proper amount of light in every room.

- Be built and constructed within budget, code, and other constraints.

- Be environmentally responsible.

- Respond to the Architecture and Interior Design

- Produce good color

- Achieve the desired moods of each space

- Be able to control the lights

# 2  Light Sources

- Incandescent

- Halogen

- Fluorescent

- Compact

Fluorescent Lamps

- LED’s

- Fiber Optic Sources

# Incandescent Lamps

•Appealing

•Warm, attractive color

(2600-2900K)

•Dimmable

•Wide variety

•Not energy efficient

Low Voltage Incandescent

## Lamps

PAR36 Long

- Usually low throw display wattage but not lamp 2700K

inherently energy efficient

Xenon incandescent strip lights

Greatest advantage:  sizeLow voltage

strips and tubes 2400-

2600K

2800K

# Halogen Lamps

•Appealing

Halogen PAR 20, PAR 30

•Crisp warm, attractive and PAR 38 lamps color (2800-3100K)

•Dimmable

•Wide variety

•Longer life, slightly

more energy efficient	Halogen lamps for table lamps and

chandeliers

Small halogen lamps for task lights, sconces, etc.

## Low Voltage Halogen

- Compact, bright lamps

PAR36 and

AR111

halogen

long throw

lamps

A bit more energy efficient

- Excellent reading, work and display light sources

- Color Temp:  2900-3100K

- Long life

MR16

# Linear Fluorescent

•T-12 “fat tubes”	T-12       T-8     T-5    T-2    no longer a good choice

•Use T-8 lamps for most residential uses

•Consider T-5 and T-2 lamps for undercabinet fixtures

Compact

# Fluorescent

•7,9, and 13 watt twin tubes (“PL” lamps) for step lights, low level lighting

•Triple tube,

Circuline and 2D lamps 18-58 watts for general lighting

# Color temperature

Range of

Daylight

Warm	Neutral	Cool	Cold

1800K          2800K        3500K         5000K           7500K 	10000K

9000

K

Setting

Sun

North

Sky

Range of

Fluo

rescent

Range

of

Flame

Range of

Incandescent

and Halogen

2200K          3000K        4100K         6500K

Recommended Fluorescent

# Lamps for Common Use

| Desired Lamp Color - Warm 2700K - Warm 3000K - Neutral 3500K - Cool 4100K - Cold 5000K | Lamp Color Designation - “827” - “830” or “930” - “835” - “841” - “850” or “950” |
| --- | --- |

Example:  F32T8/830 is a 3000K, 32 watt tubular fluorescent lamp 8/8” in diameter

# Fluorescent Ballast Technology

•Non-dimming

electronic ballasts

- Quieter – Instant starting

•Dimming

electronic ballasts

- Two wire versions now available

- Standard 3 wire and 4 wire versions also available

# New Technologies

•Fiberoptics

•LED

# Fiberoptics as Special Effect

•End-emitting fiber

- twinkle effects (star fields)

- in-water effects (ponds and water features)

•Side Emitting fiber

- outlining

- shapes

# Fiberoptics as Display Lighting

- Uses special optical elements (small lenses)

- Each fiber emits a very small amount of light

- Maximum bundle is about

300 cd at 30 degree

- Virtually no UV or IR

- Requires a low ambient light space like a fine museum

# LED Lights

- Promising technology

- Used for traffic signals and exit

signs

- New products include sconces, step lights and marker lights

For the Kitchen and Bath

## My favorites

Kitchen

- PAR38 halogen work area downlights

- MR16 low voltage accent lights

- Low voltage xenon or fluorescent undercabinet lights

- Fluorescent cove lights

Bath

- Halogen or fluorescent vanity

lights

- MR16 vanity downlight and accent lights

- MR16 shower light

# Urban Living Trends

•Smaller, higher quality spaces

•Green design

## Challenges of Green Design

- Too much emphasis on compact fluorescent lighting

- Little understanding of lighting design by utility and energy efficiency advocates

- Not many good choices of luminaires embodying green principles

- Poor design sensitivity in the green community

For the Kitchen and Bath My green favorites

Look for US EPA Energy Star but be picky!

Kitchen

- Compact

fluorescent work area downlights

- MR16 low voltage accent lights

- Low voltage fluorescent undercabinet lights

- Fluorescent cove lights

Bath

- Fluorescent vanity lights

- MR16 vanity downlight and accent lights

- MR16 shower light

## 3   Luminaires

- “Architectural”

  - Recessed cans

  - Track

  - Coves and undercabinet lights

- Decorative

  - Chandeliers

  - Sconces

  - Pendants

  - Lamps

- Utility

  - Drums

  - Closet lights

Recessed Lighting

- Generally inexpensive

- Very popular

- Aesthetically “neutral” • Good for task lighting

- OK for general lighting

- If chosen correctly, excellent for display lighting and a number of special applications

# Recessed Lighting

- The “Housing”

  - For most residential use, fixtures are at most about 7” tall for

2x8 construction

- The “Trim”

  - 4”, 5” and 6” incandescent

  - 3”, 4”, 5” and 6” low voltage

  - 4”, 5” and 6” compact fluorescent

# Standard Housings

- Incandescent non-IC

- Incandescent IC

- Incandescent air tight IC

- Low voltage non-IC

- Low voltage IC

- Low voltage airtight IC • Compact fluorescent non-

## IC

- Compact fluorescent IC

- Compact fluorescent airtight IC

# Remodeler Housings

•Designed to fit through a single hole in the ceiling

•Usually not IC

# Premium Housings

•Superior quality

•Interchangeable

lamp capability

- Incandescent

- Low voltage – Compact fluorescent

•High quality

interchangeable trims

# Choosing Trims

- Is the trim suitable for your needs

  - Adjustable?

  - Right style?

  - Right color or material?

- Is the trim LISTED for the application?

  - Kitchen:  indoor, dry location

  - Bathroom:  indoor, dry location

  - Over shower or tub:  Spa or shower rated

  - Steam shower:  wet label, gasketed

  - Indoor pool or hot tub:  wet label, non conductive trim, at least 7.5’ above water level

# Standard trims

- Downlight

  - Baffle

  - Cone

- Accent light

  - Gimbal – Eyeball – Pull down

- Wallwash

  - Eyelid

## Using Standard Recessed Lighting

• Choose trims tastefully • Use halogen lamps

- 4” family use PAR20

- 5” and 6” family use

PAR30

- 6” family also consider the

PAR38

- Avoid so called line voltage MR16 and PAR16

Adjustable

Line Voltage

Recessed

Trims

## Low Voltage MR-16 Lighting

- Typically used in a 3” or 4” recessed can

- Can be used (with the right trim) in a 5” or 6” can

- Use good quality MR16 lamps

- ALWAYS use a soft focus spread lens (Halo L111)

- Current Favorite:  Sylvania 37MR16/IR

Low Voltage Trims

# Other Low Voltage Trims

•Glass trim, gasketed for wet environments

•Low cost spa and shower

light for tubs and shower

stalls

Be Careful with Compact

## Fluorescent Downlights

- Square: not IC or AT but looks nice

- Round IC – way tall

- To get an airtight IC you are limited to 13 watts

- In the kitchen downlight you need 32 watts

Track

- Still used in museums for good reason

- Permits maximum flexibility

- Luminaires equipped easily with spread lenses and UV filters for artwork lighting

- Attaches to surface, permits dramatic lighting in condos

# Track

- Standard 120 volt track

  - Low cost general purpose

  - High cost “museum grade”

- Low voltage track

- Low voltage specialty systems

  - Two cable systems

  - Two rail systems

  - Two conductor “bars”,

“rods”, and ribbons

## Low Voltage Lighting

•Most recessed low voltage lights have a

transformer in the housing

•Most low voltage strips and some recessed housings can be connected to a remote transformer

## Accent Lighting Using Low Voltage

# Beam Quality

An unfiltered lamp tends to have

- Striation

  - lines and harsh edges

- Halation

  - rings sometimes with rainbowing

- Sharp edges and rapid change

  - well defined round or elliptical beams

# Taming the (MR16) Beast

- Smooth the Beam

  - Softening Lens (Halo L111 or “solite”) – Spread Lens

  - Linear Lens

- Shield the Source

  - Baffle or snoot

  - Louver

# Accent Lighting Technique

- Should be located at about 30 degrees off vertical relative to focal point

- Do not get too close to wall normally 24” minimum away

- Use 30-60-90 triangle to determine optimum position

- Only use lighting systems capable of hitting above 40 degrees (off vertical) in special situations.

# Accent Lighting

Artwork

30

-

60

-

90

triangle

Mounting Line

Recessed accent light

Track or monopoint

The distance from the

fixture to the artwork

will determine

beam angle and

candlepower

# Accent Lighting with PAR36

lamp

For high ceilings use a low voltage 6” recessed luminaire and a 50 watt PAR36 low voltage

# Accent lighting

- Living room, great room, family room

- Dining room

- Foyer

- Hallways/prime art locations

- Art niches

- Master bedroom

reading and art lights

- Powder rooms

# The Layered Approach to Lighting Design

Begin by thinking in layers

Decorative

Focal

Task

Ambient

# Layer #1 - The Ambient or General Lighting Layer

- Generally the relatively uniform lighting of the space.

- Tends to establish mood.

- Includes uniform downlighting,  indirect lighting (uplighting and wallwashing), and some special techniques, but can also be the decorative lighting

- Called “general lighting” if at task levels (30-50 fc or more).

- Called “ambient lighting” if lower than task levels

# The Ambient Layer

Cove lighting

# Layer #2 - The Task Layer

•Generally limited to “task lighting” of the HORIZONTAL WORK SURFACE at work locations.

•Tends to help create drama.

•Usually produces 50 fc or more within a small area.

# Ambient and Task Lighting

Downlighting

# Layer #3 - The Display or Focal Layer

- Generally limited to accent lighting and similar effects, primarily through

VERTICAL AND OTHER NONHORIZONTAL SURFACE ILLUMINATION.

- Tends to create drama, with greater drama the result of greater contrast between the brightness created by Focal Lighting and Ambient Lighting.

- Usually involves key displays at 100 fc or more.

# Ambient, Task and Focal Lighting

Accent Lighting

# Layer #4 - the Decorative or Traditional Layer

- In general,  adds the decorative luminaires called for by the architecture/interior design style, period, motif.

- Is usually expected to contribute to the ambient illumination.  In many designs, the decorative lighting will BE the ambient lighting.

- Usually reduces contrast (drama).

# Ambient, Task, Focal and Decorative Lighting

Chandelier

# Composition

# Composition

•Chandelier (ambient and decorative)

•Recessed (task)

•Recessed

(focal)

•Sconce

(decorative)

# Composition

•Cove (ambient)

•Recessed (task)

•Undercabinet

(task)

•Chandelier

(decorative)

•Sconce

(decorative)

# Composition and Planning for Change

•Recessed adjustable lighting

•Combined with fixed predictable lighting

# Remote transformers

•Address Article 411 and 725 issues

- Class 1

- Class 2

•Low noise, minimum heat

# Wallwashing

Produces a flat, even wash of light for a wall.  You may find this is NOT what

you wanted…

# Types of Wallwashers

Rececssed asymmetric style

Eyelid style

Recessed lens style

Track and surface lens and asymmetric style

Semi-recessed lens and open asymmetric style

# Principles of Wallwashing

- Fixtures at least 24” out from wall and

about 1/4 wall The higher thewall the further

height out from wallout to locate

- Fixtures apart 1 to wallwashers

1.5 times the distance from the wall

- If you don’t need at 	least three - you 	At least 2’ shouldn’t be wallwashing

## Undercabinet Lighting

DON’T

- Mount fixtures at back of cabinet

- Use luminous front fixtures

- Put short fixtures under long cabinets

- Put in cheap cool white or daylight lamps

## Undercabinet Lighting

DO

- Use solid front fixtures

- Use a lens to diffuse the lamp image over polished countertops

- Choose fixtures to be as continuous as possible and wrap around corners

# Low Voltage Specialty Lighting

- Permits finely detailed lighting effects

- Use inside and under cabinets and similar locations

# Example:  Bookcase

## Example:  Cabinet with Polished Countertop

# Things to Remember:  Room by Room

•Here is my basic list of lighting considerations

•I use this with every client – we expand from there

# Kitchen

•Multiple layers permits multiple

moods

## Steps to Better Kitchen Lighting by Layers

Layer #1 –

L

ight

the Task

## Steps to Better Kitchen Lighting by Layers

Layer #2 – Ambient

Light

## Steps to Better Kitchen Lighting by Layers

Layer #3 – Decorative

Light

# Bathroom

Layers

•Multiple layers for mood and

flexibility

Bathroom

Layer #1-Task

Light the Face

Bathroom

Layer #1-Task

Light the Face

# Bathroom

Layer #2-Task

Light the Sink

# Bathroom

Layer #3-Task

Light the

Shower or

Tub

Bathroom

Layer #4-

N

ight

Lights

# Other Lighting Things to Remember

- Dimmers and 	• Controlled controls laid out 	daylight well and kept 	• Lens, focus and

simple	other

- Layers in every 	adjustments must space even 	be done closets	• Good lighting

- Built in night lights	extends outdoors

# Lighting Design Basics

## Presented by BENYA LIGHTING DESIGN

For a copy of these slides, www.benyalighting.com